Point Cloud Processing Software

User Manual

2026-2-8

Overview

This software provides a suite of point cloud processing tools. Each script operates independently and can be run separately. All scripts use standard or common libraries (plyfile, numpy, open3d, tkinter, os).

General Workflow

- File Selection

  - Each script launches a file selection dialog upon execution.

  - Only .ply files are supported.

- Output Location

  - Processed files are saved in the same directory as the input file.

  - Output filenames follow the pattern: [original_filename]_[suffix].ply.

- Visualization

  - Where specified, visualization is enabled by default.

Module Descriptions

1. Bounding Box Cropping

Script: Box_process.py

Purpose:
Crops a point cloud using a user-defined bounding box.

Parameters:

- xl: Left bound (X-min)

- xr: Right bound (X-max)

- yl: Left bound (Y-min)

- yr: Right bound (Y-max)

- zu: Upper bound (Z-min)

- zd: Lower bound (Z-max)

Process:

- Select a .ply file via dialog.

- Enter the six bounding box parameters.

- The point cloud and bounding box are displayed (visualization is default).

- Points inside the box are retained.

Output:
[input_filename]_Boxrocess.ply

2. ExG-based Color Filtering

Script: Color_exg.py

Purpose:
Filters points based on the Excess Green (ExG) vegetation index.

Input Requirements:
.ply file must contain the following fields (in order):
x, y, z, nx, ny, nz, r, g, b
(where r, g, b are in columns 6, 7, 8)

Formula:
ExG = 2 * G - R - B

Threshold:
Default threshold = 40. Points with ExG > threshold are kept.

Process:

- Select a .ply file.

- The script calculates ExG for each point and applies the threshold.

Output:
[input_filename]_ExGFiltered.ply

3. Statistical Outlier Removal

Script: Denoise_process.py

Purpose:
Removes noise/outliers using Open3D’s remove_statistical_outlier.

Parameters:

- nb_neighbors (stnb): Number of neighboring points used for statistics.

- std_ratio (stra): Standard deviation multiplier threshold.

Algorithm:
For each point, the average distance to its nb_neighbors is computed. Points whose distance is greater than mean + std_ratio * std_dev are removed.

Process:

- Select a .ply file.

- Manually input nb_neighbors and std_ratio.

Output:
[input_filename]_StatDenoised.ply

4. Voxel Downsampling

Script: Voxel_downsample.py

Purpose:
Reduces point density using voxel grid downsampling.

Parameters:

- voxelvalue: Voxel size (edge length in point cloud units, e.g., meters).

Algorithm:
The space is divided into voxels of size voxelvalue. Each voxel retains a single representative point.

Process:

- Select a .ply file.

- Manually input voxel size.

Output:
[input_filename]_Voxel{voxel_cm}cm.ply
(Note: voxel_cm is the voxel size converted to centimeters for the filename.)

5. DBSCAN Clustering

Script: DBS_process.py

Purpose:
Performs density-based clustering using DBSCAN and filters clusters by size.

Parameters:

- eps: Neighborhood distance.

- min_points: Minimum points to form a cluster.

- limit: Minimum cluster size to retain (default example: 100).

Intelligent Filtering:

- Clusters with fewer than limit points are removed.

- If all "large" clusters together contain fewer than limit points, the largest cluster is retained (even if ≤ limit) to avoid empty results.

Process:

- Select a .ply file.

- Input eps and min_points.

Output:
[input_filename]_DBSCANFiltered.ply

6. Single-Plant COV Computation

Script: compute_COV.py

Purpose:
Computes the Coefficient of Variation (COV) for individual plants.

Note:
This module is designed for post-processing after clustering or segmentation. It calculates the COV (standard deviation / mean) for geometric or spectral attributes of each isolated plant point cloud.

Process:

- Select a clustered or segmented .ply file.

- The script computes the COV per cluster/plant.

Output:
Results are printed or saved as specified in the implementation (exact output format depends on internal script logic).

Notes

- All modules assume .ply files contain the required attributes (e.g., color channels for Color_exg.py).

- Visualization (where applicable) can be disabled by modifying the script.

- Default threshold/parameter values are examples; users can adjust them during execution.

Methodology

3D Point Cloud Analysis Pipeline for Arabidopsis Thaliana

The analysis of Arabidopsis thaliana three-dimensional (3D) point clouds was performed using a custom processing pipeline comprised of six sequential modular scripts. Each module, developed in Python, addresses a specific processing task, and their combined execution facilitates the extraction of quantifiable plant structural traits from raw 3D scan data. The pipeline leverages standard libraries including open3d, numpy, and plyfile for core computations and tkinter for graphical user interaction. The general workflow for any input .ply file involves interactive file selection via a dialog box, parameter configuration (either predefined or user-input), processing execution, and automatic saving of the output file in the same directory as the input, with a descriptive suffix appended to the original filename.

1. Data Loading and Preprocessing
Individual .ply files containing plant point clouds were loaded into the pipeline. Each point within the cloud was defined by nine attributes: three spatial coordinates (x, y, z), three normal vector components (nx, ny, nz), and three RGB color values (red, green, blue), corresponding to columns 0–8, respectively, in the data array.

2. Region of Interest Delineation via Bounding Box Cropping
To isolate the primary plant structure from the background and scanning artifacts, a 3D bounding box crop was applied using the Box_process.py module. The cropping region was defined by six parameters: left (xl), right (xr), and lower (yl), upper (yr) bounds in the horizontal plane, and down (zd), up (zu) bounds in the vertical axis. The module provided immediate visual feedback by rendering the original point cloud superimposed with a wireframe representation of the defined bounding box. All points residing outside the defined Cartesian volume {x ∈ [-xl, xr], y ∈ [-yl, yr], z ∈ [zd, zu]} were subsequently removed. The cropped point cloud was saved for subsequent analysis.

3. Vegetation Segmentation Using Excess Green (ExG) Index
Following spatial cropping, a color-based segmentation step was performed to distinguish photosynthetic plant material from non-vegetative components (e.g., soil, potting medium, labels). The Color_exg.py module calculated the Excess Green (ExG) index for every point using the formula: ExG = 2 × G - R - B, where R, G, and B are the red, green, and blue color channel intensities, respectively. A fixed global threshold (default = 40) was applied, whereby points with an ExG value greater than this threshold were classified as vegetation and retained. This process effectively filtered the point cloud to primarily green plant tissues.

4. Noise Reduction and Data Reduction
The segmented point cloud underwent two successive cleaning and simplification operations. First, statistical outlier removal (Denoise_process.py) was employed to eliminate spurious noise points (e.g., sensor errors, floating pixels). This algorithm computed the mean distance (mean_dist) and its standard deviation (std_dev) from each point to its nb_neighbors nearest neighbors. Points whose mean distance exceeded mean_dist + std_ratio × std_dev were classified as outliers and removed, where std_ratio is a user-defined multiplier.
Subsequently, voxel grid downsampling (Voxel_downsample.py) was applied to uniformly reduce point density while preserving the overall geometric morphology. The 3D space was subdivided into a grid of cubic voxels with a user-specified edge length (voxel_size in cm, internally converted to meters). Within each occupied voxel, all points were replaced by a single representative point (their centroid), significantly decreasing computational load for subsequent steps.

5. Individual Plant Instance Identification via DBSCAN Clustering
To separate individual plant instances or major plant components within the processed scene, density-based spatial clustering (DBSCAN) was implemented in DBS_process.py. The algorithm grouped points based on a neighborhood distance (eps) and a minimum point threshold per cluster (min_points). A post-clustering filter was applied to retain only significant clusters: primarily, clusters containing more than a minimum point count (limit) were kept. A fallback mechanism ensured a result was always generated; if the total points from all "large" clusters remained below the limit, the single largest detected cluster was retained to prevent an empty output.

6. Canopy Coverage and Volume Estimation
Finally, the compute_COV.py module was executed to estimate a canopy-scale structural metric from the clustered point cloud. The processed point cloud for a single plant was first spatially normalized (centered and rotated to a standard orientation). A synthetic canopy was then simulated by replicating this normalized plant point cloud in a grid pattern (default: 4×4) with random per-plant variations in height (drawn from a truncated normal distribution) and rotation around the vertical axis. From this simulated canopy, a central region was extracted to avoid edge effects. A Coefficient of Volume (COV) metric was computed by performing a final voxel downsampling (voxel size = 0.1 m) on the central canopy points and calculating the total occupied volume as: COV = N × v³, where N is the count of downsampled points and *v* is the voxel edge length. This COV value, representative of the plant's volume-filling capacity within a defined canopy space, was recorded for each input file in a summary CSV file.

Methods

3D Point Cloud Analysis Pipeline for Arabidopsis Thaliana

This study established a computational pipeline for analyzing 3D point clouds of Arabidopsis thaliana, integrating six sequential processing modules. Each module was implemented as a standalone Python script utilizing standard libraries (open3d, numpy, plyfile).

The raw .ply file, containing point coordinates, normals, and RGB color data, was first cropped to a defined bounding box (Box_process.py) to isolate the plant. Vegetative pixels were then segmented using the Excess Green index (ExG = 2G – R – B) with a fixed threshold of 40 (Color_exg.py).

The resulting point cloud was denoised by removing statistical outliers based on neighborhood distances (Denoise_process.py) and uniformly downsampled via voxel grid simplification (Voxel_downsample.py). Individual plant instances were subsequently identified and isolated using DBSCAN density-based clustering with a post-filter to retain only substantial clusters (DBS_process.py).

Finally, for volume estimation, a normalized plant point cloud was used to generate a synthetic 4x4 canopy grid with randomized height and rotation. The central region of this canopy was extracted, and a Plant Occupation Volume (POV) was calculated from a voxel-downsampled representation (compute_COV.py).